Configuring through Telnet


[image: image1.jpg]voice




TELNET COMMANDS GUIDE

FOR SWISSVOICE IP PHONES

Edition 3.0

Contents

	Contents


3About this guide

Introduction
3
Conventions
3
Related documentation
3
Abbreviations
4
General Telnet commands
5
Password and login configuration (Telnet and Web)
10
DSP related configuration
11
Setting telephone book
13
Setting Idle screen
15
Set language
15
Show language name
16
Administrator password on local MMI
16
Utilities
17
Dim Ver
19
Setting call agent localization and MGCP parameters
19
Features and supplementary services
25
Setting function keys
28
RFC3149 configuration commands
29
Commands to set tone table elements and ringing tables
32
Some other commands
35
Setting SIP parameters
37
Setting overall parameters (SIP version)
40
Setting H323 parameters
41
Setting overall parameters (H323 version)
43
Setting codec parameters (H323 and SIP versions)
44
Setting numbering table (H323 and SIP versions)
48
ANNEXE A:configip10.cfg sample for SIP
50


 About this guide

	Introduction


	Conventions


Typographical conventions

Typographical conventions used throughout this guide are :

Boldface

Names of items displayed in the phone. 

Red color

Variables to be set.


	[image: image2.wmf]
	Note: Contains helpful suggestions or information not covered in the document.


	Caution
	Caution: Means that the reader should be careful. In this situation, you might do something that could result in phone damage or loss of data.


	Related documentation


For more information about using Swissvoice products, refer to one or more of the following publications:

· Phone user guide

· MGCP XML manual

· Administrator guide

When available, above publications can be downloaded from Swissvoice website:

www.swissvoice.net – Worldwide – Support – Manuals

About this guide

	Abbreviations


DNS
Domain Name Server

DHCP
Dynamic Host Configuration Protocol

FTP
File Transfer Protocol

H. 323
An ITU standard for realtime voice and videoconferencing over packet networks,


including LANs, WANs and the Internet

LAN
Local Area Network

MGCP
Media Gateway Control Protocol. An IP telephony signaling protocol from the


IETF

MMI
Man Machine Interface
RFC3149
Request For Comments. This document describes the specifications for business


extended services under the MGCP protocol

RTCP
Real Time Control Protocol

RTP
Real Time Protocol
SIP
Session Initiation Protocol.  An IP telephony signaling text-based protocol


developed by the IETF 

SNTP
Simple Network Time Protocol
TFTP
Trivial File Transfer Protocol

TOS
Type of Service
XML
EXtensible Markup Language

	General Telnet commands


The phone can be configured using any telnet terminal. All telnet commands can be part of a script file (.cfg file) that can be automatically downloaded during the phone boot process (see "Automatic phone provisioning", page …).

Starting a Telnet session

1. Start a Run session on Windows.

2. In the Open field, Key in telnet followed by the IP address of the phone. 

  For example : telnet 10.3.16.110
3. Key in following login and password when requested :

VxWorks login: target
Password: password
->

	[image: image3.wmf]
	Login and password can be changed (see "Passwords", page). If forgotten, the only way to retrieve the default telnet login is to log as administrator under web browser and to make a reboot with option Restore factory settings activated.


General commands

Table 1 lists general telnet commands.  

Table 1

	Prompts
	Results

	->
	Operating system prompt level



	MXP>
	Application prompt level

	Commands to move from one level to the other:



	dbgcmd
	To move from the operating system level «->» to the application level« MXP> »

	exit
	To move back from application level to operating system level


Viewing parameters settings under application level 

Table 2 lists commands to view parameters included in the IP phone application.

Example: to see the current configuration of TCP/IP parameters, enter the command :

MXP>show configure
The answer is given as follow :

MXP>show configure

Flash

 configure ftp_filename        = 'flash'

 configure ftp_username        = 'ip10_91'

 configure ftp_userpassword    = 'ip10_91'

 configure ftp_addr            = 10.3.10.31

 configure cfg_filename        = 'configip10.cfg'

 configure tftp_addr           = 0.0.0.0

 configure inf_filename        = 'swupdate_ip10.inf'

 configure DHCP                = 0

 configure ip_addr             = 10.3.16.118

 configure subnet_mask         = 255.255.0.0

 configure gw_addr             = 0.0.0.0

 configure dns_addr            = 255.255.255.255

 configure VLAN ID             = 65535

 configure VLAN TX             = 0

 configure VLAN priority       = 255

 configure IP TOS byte         = 0

MXP>

To change these parameters, you can get the commands syntax by entering "set configure":

MXP>set configure

BOOT PARAMETERS CONFIGURATION COMMANDS:

set configure ftp_filename [FTP file name]

  set configure ftp_username [FTP user name]

  set configure ftp_userpassword [FTP user password]

  set configure ftp_addr [ X.X.X.X ]

  set configure cfg_filename [cfg_filename]

  set configure tftp_addr [ X.X.X.X ]

  set configure inf_filename [inf_filename]

  set configure ip_addr [ X.X.X.X ]

  set configure dhcp [ 0 = no, 1 = yes ]

  set configure subnet_mask [ X.X.X.X ]

  set configure gw_addr [ X.X.X.X ]

  set configure dns_addr [ X.X.X.X ]

  set configure vlan_id [ value ]

  set configure vlan_tx [ 0 = no, 1 = yes ]

  set configure vlan_pri [ 0( low ) thru 7(high) ]

  set configure ip_tos

MXP>

The parameter "ftp_filename" must stay to "flash" value. It means that when booting, the application will be taken from flash location. If the value is something else, then the phone will try to download this filename through FTP, but will not save it into flash memory.

Once you modified all needed parameters with command "set configure …", you need to save these new values into flash memory, with the command "commit configure".

MXP>commit configure
MXP>
Once saved into flash, some of these parameters need the phone to be restarted in order to be taken into account. These parameters are :

· ip address of the phone (ip_addr),

· subnet mask,

· dhcp use (dhcp),

· gateway ip address (gw_addr),

· VLAN configuration,

· type of service (ip_tos).

Table 2

	Commands to show current parameters values under application level

	MXP>show <command>
	Show types of parameters 

	MXP>show configure
	Display the current configuration of TCP/IP parameters

	MXP>set telnet
	

	MXP>show telbook
	Show contents of the current phone book, sorted by alphabetic order.

	MXP>show idle_screen
	Show the idle text of the display.

	MXP>show language
	Show language parameters.

	MXP>show language name
	Show language list.

	MXP>show local_password
	Show the password used to go into the phone « administrator settings ».

	MXP>show utilities
	Show the current configuration of following parameters : SNTP, tone table, web activation

	MXP>dim ver
	Show the current version of DSP image.

	MXP>show tcid 0
	

	MXP>show xgcp
	

	MXP>set xgcp
	

	MXP>show features
	

	MXP>show function key
	Show current function keys configuration.

	MXP>show rfc3149
	

	MXP>show tone
	


Changing parameters settings under application level 

To modify any setting, the syntax  “set <command> " is used. 

With this type of command, you are able to change following parameters:

· tcp/ip parameters,

· call agent location,

· telephone book,

· utilities,

· some other commands.

Once a parameter is changed, it must be activated and saved into flash memory. 

Commands are:


MXP>activate

MXP>commit

Moreover, for some parameters, the phone needs to be restarted under operating prompt level (see table 3, page ).


->reboot
	[image: image4.wmf]
	When using a script to configure several parameters, always wait for the prompt ("->" or "MXP>") after the activation of a command before entering a new command, else this command will not be taken into account.


TCP/IP parameters

1 - To change TCP/IP parameters, you can get the command syntax by entering:

MXP>set configure

BOOT PARAMETERS CONFIGURATION COMMANDS:

  set configure ftp_filename [FTP file name]

  set configure ftp_username [FTP user name]

  set configure ftp_userpassword [FTP user password]

  set configure ftp_addr [ X.X.X.X ]

  set configure cfg_filename [cfg_filename]

  set configure cfg_filename_2 [cfg_filename]

  set configure tftp_addr [ X.X.X.X ]

  set configure tftp_addr_by_dhcp [ 0 = no, 1 = yes ]

  set configure inf_filename [inf_filename]

  set configure inf_filename_by_dhcp [ 0 = no, 1 = yes ]

  set configure ip_addr [ X.X.X.X ]

  set configure dhcp [ 0 = no, 1 = yes ]

  set configure subnet_mask [ X.X.X.X ]

  set configure gw_addr [ X.X.X.X ]

  set configure dns_addr [ X.X.X.X ]

  set configure vlan_id [ value ]

  set configure vlan_tx [ 0 = no, 1 = yes ]

  set configure vlan_pri [ 0( low ) thru 7(high) ]

  set configure ip_tos

MXP>

Then, change values inside square brackets to your convenience.

	[image: image5.wmf]
	The parameter "ftp_filename" must stay to "flash" value. It means that when booting, the application will be taken from flash location. If the value is something else, then the phone will try to download this filename through FTP, but will not save it into flash memory.


	[image: image6.wmf]
	The parameters "tftp_addr_by_dhcp " and "inf_filename_by_dhcp" allow, when set to "0", not to take care of options 66 and 67 given by DHCP server. It means that configured values for "inf_file_name" and "tftp_addr" are not overwritten by DHCP.


2 – Save new values into flash memory:

MXP>commit configure
MXP>

3 – Restart the phone:

Once saved into flash, some parameters need the phone to be restarted in order to be taken into account. These parameters are :

· ip address of the phone (ip_addr),

· subnet mask,

· dhcp use (dhcp),

· gateway ip address (gw_addr),

· VLAN configuration,

· type of service (ip_tos).

The reboot of the phone has to be done under operating prompt level:

MXP>exit
->

->reboot

Password and login configuration (Telnet and Web)

MXP>set telnet

Use the following command to set the telnet login/password

  set telnet login <login> 

  set telnet password <password (8-20 characters)>

These commands need 'activate' + 'commit' for changes to be saved...!

OK

MXP>

MXP>set webaccess
COMMANDS TO SET WEB ACCESS :

set webaccess <USER|ADMIN> <LOGIN|PASSWORD> <data>

MXP>
Then you need to activate the new configuration and to save it into flash :

MXP>activate

OK

MXP>commit

OK

MXP>

	[image: image7.wmf]
	if you don’t remember the telnet login and password you configured, and you left the telnet session, then the only way to go back to initial configuration (« target » and « password ») is to restore factory seetings through Web interface (log as administrator) and to reboot the phone.


DSP related configuration

DSP configuration is related to voice parameters, and also some specific signals configuration. This type of configuration regards MGCP version. Description of H323 and SIP version DSP configuration is done later in this document.

Voice activity detection :

Some commands allow to configure voice activity detection parameters. Activation or deactivation of voice activity detection is made through command :

MXP>set coding 1 vad [on|off]
This command apply to all codecs (G711, G729, G7231).

Once VAD is activated, you can configure the audio threshold level for detection of voice (which is the signal energy level that distinguishes "valid" sounds from background) :

MXP>set coding 1 vad_thresh <value -20 to +10, 32767 for adaptive VAD>
This command only applies with G711, as other codec (G729b and G723.1a) manage their own algorithm.

MXP>set tcid 0 idle_noise <signed valule in .01 dB>
This command is used to set the Idle Noise level (also known as “comfort noise”).
Jittering configuration :

MXP>set coding 1 adaptive_playout [off|adj_silence|adj_immed]
This command is used for the playout buffer. When you do a 'set coding 1 adaptive_playout on' then the the IP10S set minimum, maximum and nominal values for the playout buffer (depending on codec and framelength). The nominal value is only used for the first time the voice is played out in a voice session. Once the playout is started then the DSP will try to calculate new nominal values to use and will assign new nominal values based on its network calculations. In this case the newly calculated nominal value cannot go above the maximum value and cannot go below the minimum value the IP10S have already set. 'adaptive_playout adj_silence' indicates that the current nominal value needs to be adjusted with the newly calculated nominal value at the next silence period. 'adaptive_playout adj_immed' indicates that the current nominal value needs to be adjusted with the newly calculated nominal value immediately.

DTMF relay configuration :

DTMF relay can be configured "out of band" (protocol signalisation), "inband" (RFC2833) or "off" (digit coded inside voice). Depending on MGCP call flow, the behaviour can change independantly of the configuration. 

MXP>set coding 1 dtmf_relay <on|off> [inb|oob|inbm|oobv]
This command is subect to evolve.

You can supervise the digit you dial by 2 ways : under key press, a local DTMF digit is played locally (configuration "off"), or a beep (the same one for all digits) is played locally (configuration "on").

MXP>set tcid 0 ipp kpad_echo <on|off>

	[image: image8.wmf]
	For MGCP protocol, to get DTMF relay activated, it's also needed to configure "set tcid 0 ipp kpad_echo off".


Specific parameters and configuration for MGCP:

The dtmf relay will work only if no digits are requested through RQNT message (R:[0-9*#]).

It's also possible to activate some patches, to force a behaviour of the phone inside a given mode :

MXP>set utilities patches dtmf_voice on
Then DTMF will be always send inside voice (normal RTP packets) and eventually doubled inside mgcp signalisation (if R:[0-9*#] asked by the call manager). It seems that it is what you want/need.

MXP>set utilities patches dtmf_RFC2833 on
Then DTMF will be always send according to RFC2833 (normal RTP packets) and eventually doubled inside mgcp signalisation (if R:[0-9*#] asked by the call manager).

In all case the phone send DTMF according to RFC2833, the default payload is 97, but will be changed accordingly to SDP parameters received from the far end.

Call waiting configuration

Number of waiting tone signal to be played can be 1 or 2, and can be configured with :

MXP>set tcid 0 cw_attempts <1 or 2>
When cw_attempts is "2", it's then possible to define the intercycle between the 2 signals are played, with following command :

MXP>set tcid 0 cw_resend_time <time in sec>
Setting telephone book

Both private and common phone books can be configured through telnet.

Compared to previous releases, it is now possible to have the common phonebook on a server on the LAN, instead of locally in the phone.

To configure the phonebook, the following commands are available (enter "set telbook" to see them):

MXP>set telbook

Use one of the following commands

  set telbook add internal/private "<Name>" "<Number>"

  set telbook del "<Name>" "<Number>"

  set telbook del internal/private/all

  set telbook CommonTelBookSource Local/Remote

  set telbook RemoteServerAddress <IP address|DNS address>

  set telbook RemoteServerPort <Port [1024..65535]>

  set telbook RemoteTelbookPath <Path>

These commands need 'activate' + 'commit' for changes to be saved...!

OK

MXP>
A phonebook record contains a name (24 characters maximum) and a number (26 digits maximum).

Command « set telbook add internal/private "<Name>" "<Number>" » allows to add records one by one to the phonebook. If « Name » length or « number » length are too high, then number or name will be truncated to the maximum length (24 for name and 26 for number).

Command « set telbook del "<Name>" "<Number>" » allows to delete phonebook records one by one.

Command « set telbook del internal/private/all » allows to delete a comple phonebook (common, private or both) in one command.

Common phonebook location is configured with command « set telbook CommonTelBookSource Local/Remote ».

If phonebook is not local in the phone, then it’s needed to configure its location. It can be done by configuring directly IP address of « phonebook server », and giving the location of this phonebook on the server.

	[image: image9.wmf]
	when X-state and X-Features package is managed by the call manager, location of remote phonebook can be dynamically changed by the call manager using « X-directory » parameter.


To check the contents of the current phonebook, sorted by alphabetic order, enter the command "show telbook":

MXP>show telbook

Common TelBook Source : LOCAL

Remote TelBook parameters :

  ->Remote Server Address : 10.3.20.190

  ->Remote Server Port    : 8080

  ->Remote TelBook Path   : /swissvoicedirectory

Item n°1 - Private telbook - name = aaaaaa - number = 1111111

Item n°2 - Private telbook - name = bbbbb - number = 22222

Item n°3 - Private telbook - name = ccccc - number = 33333

Item n°4 - Internal telbook - name = coooooo - number = 107

Item n°5 - Private telbook - name = ddddd - number = 44444

Item n°6 - Private telbook - name = eeeee - number = 55555

Item n°7 - Private telbook - name = fffff - number = 66666

Item n°8 - Internal telbook - name = Geeeeee - number = 112

Item n°9 - Private telbook - name = ggggg - number = 77777

Item n°10 - Internal telbook - name = iiiiiiiii - number = 102

Item n°11 - Internal telbook - name = Jffffffff - number = 105

Item n°12 - Internal telbook - name = Jmmmmmmmm - number = 104

Item n°13 - Internal telbook - name = Labo VoIP - number = 100

Item n°14 - Internal telbook - name = Labo Voip2 - number = 120

Item n°15 - Internal telbook - name = Nnnnnnnnnn - number = 110

Item n°16 - Internal telbook - name = Patrice Rrrrrr - number = 111

Item n°17 - Internal telbook - name = Patrick Llllll - number = 114

Item n°18 - Internal telbook - name = Raaaaaaaaa- number = 108

Item n°19 - Internal telbook - name = Reeeeeee - number = 109

Item n°20 - Internal telbook - name = Thierry Jjjjjj - number = 101

Item n°21 - Internal telbook - name = Thierry Nnnnnn - number = 106

Item n°22 - Internal telbook - name = Uuuuuuuu - number = 113

Item n°23 - Internal telbook - name = Yyyyyyyy - number = 103

MXP>
	[image: image10.wmf]
	even when common phonebook is configured as remote, the « show » command list the local common phonebook.


Setting Idle screen

The idle screen can take different shape, to show the current configuration, enter the following command :

MXP>set idle_screen
Use one of the following commands

* For Text Interface

   --> set idle_screen <user_name/external_number/internal_number>

   --> set idle_screen free_idle "<idle text>"

* For Graphic Interface

   --> set idle_screen <time_date/operator_logo/user_logo>"

These commands need 'activate' + 'commit' for changes to be saved...!

OK

MXP>
	[image: image11.wmf]
	« user_name », « external_number », « internal_number » are available only if « X-state and X-feature » package is supported (see ref[1]).




Set language

To download a new language as a replacement of default one (spanish), you need to use the following commands :

MXP>set language
COMMAND TO SET LANGUAGE PARAMETERS:

  set language filename "<filename>"

  set language default  (Spanish)

OK

MXP>

The command ‘set language filename « langue.xml » ‘ launches a download through FTP. It uses FTP parameters configured on the phone.

Spanish language will then be replaced by the new downloaded language. To retrieve it, a « flash clean » is needed.

Show language name

MXP>show language
COMMAND TO SHOW LANGUAGE PARAMETERS:

  show language Name

  show language ExtInfoline

  show language ExtMenuline

  show language ExtLocalMonth

  show language ExtWebShortText

  show language ExtWebLongText

  show language ExtWebPopup

  show language ExtWebButton

MXP>

MXP>show language Name

Extended Language Name:

Español...

MXP>
	[image: image12.wmf]
	If a new language is needed, but not available, it’s up to Swissvoice to give new file to download, because there is a particular format to respect for each message (length, optional parameters, …). Please ask your Swissvoice representation.


Administrator password on local MMI

This password is asked in local man machine interface while going into « administrator settings ».

MXP>show local_password
Local password (used only in the local MMI): 0000

MXP>

MXP>set local_password
Use the following command to set a password of strictly 4 digits

  set local_password <number> 

These commands need 'activate' + 'commit' for changes to be saved...!

OK

MXP>

Utilities

SNTP parameters, web activation or inhibition, tone table can be adjusted. To check the current configuration, enter the following command:

MXP>show utilities

 utilities sntp_mode           = local 

 utilities sntp_srv_ipaddr     = 0.0.0.0 

 utilities sntp_time_zone      = 12 

 utilities sntp_daylightsaving = on (tone table)

 utilities sntp_rqnt_freq      = 24 hh 0 mm 

 utilities time_format         = 24h 

 utilities date_format         = DD/MM/YY 

 utilities tone_table          = EU 

 utilities headset_ring_mode   = LOUDSPEAKER 

 utilities AutoHookOn          = 3 sec. 

 utilities flashSaveDelay      = 10 sec. 

 utilities web_activation      = on

 utilities patches dtmf_voice  = off

 utilities patches dtmf_RFC2833= off

 utilities patches glare       = off

 utilities patches echo        = off

 utilities patches rtcp_bye    = off

MXP>
To set the utilities parameters use the following commands:

MXP>set utilities

UTILITIES PARAMETERS CONFIGURATION COMMANDS:

  set utilities sntp_mode [local|broadcast|request] 

  set utilities sntp_srv_ipaddr [xxx.xxx.xxx.xxx] 

  set utilities sntp_time_zone [xx] 

  set utilities sntp_daylightsaving [on|off|australia] 

  set utilities sntp_rqnt_freq [hh] [mm] 

  set utilities time_format [12h|24h] 

  set utilities date_format [ 0 = DD/MM/YY, 1 = MM/DD/YY, 2 = DD/MMM/YY ] 

  set utilities web_activation [on|off] 

  set utilities tone_table [US|EU|CONF] 

  set utilities headset_ring_mode [LOUDSPEAKER|HEADSET] 

  set utilities AutoHookOn <time in sec. 0(inactive) to 15> 

  set utilities OperatorLogo "<filename>" 

  set utilities UserLogo "<filename>" 

  set utilities patches [dtmf_voice|rtcp_bye|dtmf_RFC2833|echo|glare] [on|off] 

  set utilities flashSaveDelay <time in sec. sup 5s, 65535(inactive)> 

MXP>

The command "set utilities patches" allows to activate different specific software behaviour:

· DTMF_voice and dtmf_RFC2833 are described inside section "DTMF relay",

· Glare : when activated ("on"), this patch disable the glare condition detection,

· Rtcp_bye : when activated, it allows not to send any "RTCP BYE" on SSRC change,

· Echo  allows to use another table of gains in handset mode : the 2 highest levels are suppressed, and gains are recalculated between minimal value and nominal one.

About the command to configure time zone, you can get some help if you enter :

MXP>set utilities sntp_time_zone

set utilities sntp_time_zone [xx]

  0 -  -11 
Midway Island, Samoa

  1 -  -10 
Hawaii

  2 -  -9 
Alaska

  3 -  -8 
PST(USA and Canada)

  4 -  -7 
MST(USA and Canada)

  5 -  -6 
CST(USA and Canada)

  6 -  -5 
EST(USA and Canada)

  7 -  -4 
AST(USA and Canada)

  8 -  -3 
Newfoundland

  9 -  -2 
Mid-Atlantic

  10 - -1 
Azores,Cape Verde

  11 -  0 
Greenwich mean Time

  12 - +1 
Paris,Madrid,Berne

  13 - +2 
Athens,Helsinki,Istanbul

  14 - +3 
Baghdad,Kuweit,Nairobi

  15 - +4 
Abu Dhabi,Tbilisi,Volgograd

  16 - +5 
Islamabad,Karachi,Tashkent

  17 - +5:30 
Bombay,Calcutta,New Dehli

  18 - +6 
Almaty,Dhaka

  19 - +7 
Bangkok,Jakarta,Hanoi

  20 - +8 
Hong Kong,Perth,Singapore

  21 - +9 
Tokyo,Osaka,Seoul

  22 - +9:30 
Adelaide

  23 - +10 
Brisbane,Melbourne,Sydney

  24 - +11 
Solomon Islands,New Caledonia

MXP>

Example : if you want time zone "GMT + 1", then you have to enter "set utilities sntp_time_zone 12"

The command "set utilities flashSaveDelay <time in sec. sup 5s, 65535(inactive)>" allows  to configure the timeout, one in idle state, which activate the write into flash memory to save datas and configuration. To disable this write into flash, just use value 65535.

Then you need to "activate" and "commit" the changes.

Dim Ver

If you want to check the current version of DSP image, enter the command "dim ver" :

MXP>dim ver
DSP 0: version = Cap 0xc000, AER tail len 10, Rel 9.1.0.4, Build p8  , DSP C5472, Platform VPBX/GG6E, Cap2 0x4682, Cap3 0x2002, Chans 2, EC tail len 0, PCM chans 0, Cap4 0x0, FIFO size 200

Setting call agent localization and MGCP parameters

Call agent localization :

To check the current call agent localization, enter the following command "show tcid 0"  (only the 2 last lines (in red) show call agent configuration):

MXP>show tcid 0
Mode: Switched xGCP

        Pref Voice coding profile:   1

        Pref Fax coding profile:     0

Telephony Interface Configuration:

        Gain (RX,TX) = (0,0)

        Idle noise level = -6500 x .01 dB

Signaling Protocol: IPP

IPP Parameters:

        Ring Id: Default (-1)

        KeyPad Echo:  OFF

        Microphone gain         : 81 x .5 db

        Speaker gain    (volume): 5 x .5 dB

        Handset TX gain         : 58 x .5 db

        Handset RX gain (volume): 0 x .5 dB

        LCD gain: 6

        IPP Conference Enable: OFF

Dial Out Parameters:

        Out Wait: 250 msec

        Out Type: tone

        Pause Duration: 1000

        Tone Out Off Time: 70 msec

        Tone Out On Time: 70 msec

        Tone Out Power: -130 x 0.1 dB (not configurable)

Echo Cancellation Parameters:

        Config: AES

V.18A Tone Detect Parameters:

        Hangover: 0

        Threshold: 0

        Fraction: 0

Other Parameters

        Call Limit: 0 sec

        Endpoint id : aaln/1

        Partial Digit Timer: 0 sec

        Critical Digit Timer : 0 sec

        Service Class : 0

        Default Digit Map:

        Default Event List:

        Entity to be notified: ca@[10.3.16.190]:2427

        Backup entity to be notified: ca@[10.3.16.190]:2428

MXP>

The last 2 lines show the configuration : "Entity to be notified" show the primary call agent location and "Backup entity to be notified" is a backup call agent location.

Entity to be notified can be changed with the command :

"set tcid [tcid] notify_entity <Ip Address:port of Call Agent>":

MXP>set tcid 0 notify_entity ca@[172.16.1.190]:2427

If the port number is not specified, then we use the default one (2427).

Backup entity can be manually change with command :

“« set tcid [tcid] backup_notify_entity <Ip Address:port of Call Agent>"

The backup entity can be also dynamically updated through the MGCP call flows, if configuration is activated for that (set xgcp N_to_BackupEntity <YES|NO>). This value is modified each time the phone receives a N: parameter which is different from the primary call agent location. This is mainly used to have a backup in case of call agent failure. In that case the phone tries to send the RSIP messages on notify_entity first, then on backup entity.

MGCP configuration

MXP>set xgcp

xGCP Configuration Commands:

  set xgcp restart_wait <time in sec> 

  set xgcp re_xmit1_limit <retries> [disable]

  set xgcp re_xmit2_limit <retries> [disable]

  set xgcp init_ntfy_to <time in msc> 

  set xgcp max_ntfy_to <time in msc> 

  set xgcp size_history_list <time in sec> 

  set xgcp rgw_name <NO_NAME|IP_ADDR|MAC_ADDR|STRING> <string (< 40 chars)> 

  set xgcp ep_name <DEFAULT|MAC_ADDR|STRING> <string (< 40 chars)> 

  set xgcp epdisp_name <DEFAULT|STRING> <string (< 40 chars)> 

  set xgcp dns_ip <IP addr in '.' notation> [enable|disable]

  set xgcp dns_ip2 <secondary IP addr in '.' notation> [enable|disable]

  set xgcp ts_max <time in sec> 

  set xgcp td_init <time in sec> 

  set xgcp td_max <time in sec> 

  set xgcp td_min <time in sec> 

  set xgcp sig_tos <service class 0-255> 

  set xgcp rtcp_interval <rtcp pkt generation period(sec) [disable(65535)|enable(0..65534)]> 

  set xgcp ts_thresh <time stamp threshold for rc4> 

  set xgcp gw_recv_port <port> 

  set xgcp persistent_evt  <ACTIVATED|DEACTIVATED> 

  set xgcp default_ca_port <port> 

  set xgcp lockstep_to <time in sec, or -1 for infty> 

  set xgcp KeepAliveInterval <interval in sec(0 sec: No KEEP ALIVE|max:65535 sec)>

  set xgcp KeepAliveReact <OFF|RSIP|REBOOT> <nb of NOT ACKED keep alive (0: inactive|1-255)>

  set xgcp RsipReact <nb of RSIP received before rebooting (0: inactive)>

  set xgcp Rsip500React <nb of RSIP 500 received before rebooting (0: inactive)>

  set xgcp PackageNotify <TRUE|FALSE>

  set xgcp RtpBasePort <Port value [1024..65535]>

  set xgcp RtpDestPort <DYNAMIC|STATIC [Port value [1024..60000]]>

  set xgcp N_to_BackupEntity <YES|NO>

  set xgcp SecurityEntity <YES|NO>

MXP>

Phone identity

The phone can be identified by one of the three following parameters: a name, the mac address or the IP address.

1- Key in the command:

MXP>set xgcp rgw_name <NO_NAME|IP_ADDR|MAC_ADDR|STRING> <string(<40chars)>
“IP_ADDR” stands for IP address. E.g.: aaln/1@[10.3.16.105]), 

“MAC_ADDR” stands for mac address. E.g.: aaln/1@000590020027,

"STRING" means that the phone will be identified by a name. E.g.: aaln/1@name1.name2" or “phone1@name1.name2”.

"NO_NAME" means that the phone will be identified  by only its "ep_name".

MXP>set xgcp ep_name <DEFAULT|STRING> <string(<40chars)> 

"DEFAULT" means aaln/1,  "STRING" identify a configurable name.

2- Then you need to activate the new configuration and to save it into flash :

MXP>activate
OK

MXP>commit
RTCP/RTP configuration

Real Time Control Protocol is activated in factory settings and default interval is 15 seconds.

You may activate or deactivate the Real Time Control Protocol. (see also command "set utilities patches rtcp_bye"). Command syntax is: 

MXP>set xgcp rtcp_interval <rtcp pkt generation period(sec) [disable(65535)|enable(0..65534)]>
OK

MXP>activate

OK

MXP>commit
Sending and receiving voice is done through RTP protocol. You can configure the basic port number for reception, and also, for emission, decide if the phone is slave of SDP parameters (DYNAMIC) or if its behaviour is static (STATIC). Recommanded configuration for emission is DYNAMIC, which is the standard behaviour.

MXP>set xgcp rtcpBasePort <port value [1024..65535]>
OK

MXP>activate

OK

MXP>commit
For the first connection, configured port will be used. For a second simultaneous connection, port number +2 will be used.

MXP>set xgcp rtcpDestPort <DYNAMIC|STATIC [port value [1024..60000])>
OK

MXP>activate

OK

MXP>commit
MGCP configuration for signalisation

Default sending port number:

MXP>set xgcp default_ca_port <port>
Persistent event activation (hu, hd, hf):

MXP>set xgcp persistent_evt <ACTIVATED|DEACTIVATED>
Keepalive message for NAT traversal:

In order to keep alive NAT traversal, it's necessary to send regularly a message to the call manager. Tthis can be done through a RSIP message, like shown below :

RSIP 9425 aaln/1@[10.3.10.12] MGCP 1.0 NCS 1.0

RM: X-keepalive

It's not mandatory that this message is acknoledged by the call manager (see next parameter configuration).

MXP>set xgcp KeepAliveInterval <interval in sec(0 sec:No KEEP ALIVE|max:65535 sec)>
Keep alive response management:. When a Keep alive is sent to the call agent if there is no acknowledment,  the IP10 can have the following reaction :

· Nothing to do

· RSIP disconnected if no response after n un-acked messages

· Reboot if no response after n non-acked messages

The choice is made with:

MXP>set xgcp KeepAliveReact <OFF|RSIP|REBOOT> <nb of NOT ACKED keep alive (0: inactive|1-255)>
Behaviour on RSIP not answered : when a "RSIP:restart" is sent to the call agent if there is no acknowledment,  the IP10 can have the following reaction :

· Nothing to do

· Reboot if no response after n non-acked messages

The choice is made with:

MXP>set xgcp RsipReact <nb of RSIP received before rebooting (0: inactive)>
Behaviour on RSIP acknowledged with code 500 : when a "RSIP:restart" is sent to the call agent and is acknowledged with error 500,  the IP10 can have the following reaction :

· Nothing to do

· Reboot once n RSIP 500 acknowledgment

The choice is made with:

MXP>set xgcp Rsip500React <nb of RSIP 500 received before rebooting (0: inactive)>
Package name notification with events:

For some call managers, it's mandatory to notify events with the name of the package it belongs to. This can be activated with the following command :

MXP>set xgcp PackageNotify <TRUE|FALSE>
MGCP security :

It's possible to configure IP10 to take account only of MGCP mùessages arrivent from known call manager ip address, given through parameters "notify_entity", "backup_notify_entity" and parameter "N:" of MGCP messages.

MXP>set xgcp SecurityEntity <YES|NO>
Digit notification kind of reporting:

During dialling phase, digits can be notified to the call manager, according to a digit map, into black mode or comma separated. This can be configured with :

MXP>xgcp set_digit_report <0*=comma separated|1=block>
Default quarantine mode:

MXP>xgcp set_def_qhandle <0*=discard, 1=process, 2=step, 3=loop>
Wild card protection for endpoint name: 3 modes for wild card acceptance :

· 0 accepts *@*

· 2 accepts *@rgwname

· 3 accepts only endpointID@rgwname

MXP>xgcp set_parse_mode <0|2|3>
SDP parameters presentation

MXP>xgcp set_sdp <0=basic,1*=full>
Enabling pigybacking :

MXP>xgcp set_pb_enable <0=disable, 1*=enable>
MGCP version signalled inside MGCP messages :

MXP>xgcp set_mgcp_ver <0=MGCP 0.1, 1=MGCP 1.0>
All these parameters need "activate" + "commit".

Features and supplementary services

For some Call Agents which do not manage the Swissvoice MGCP extension package, you have the possibility to change some feature activation codes.

To check the current feature configuration, enter the command "show features":

MXP>show features
show result of 'set features' command

+------------+-------------------------------------+--------+------------+

| feature ID | feature name                        | active | seq        |

+------------+-------------------------------------+--------+------------+

| CFU_ON     | Call Forward Unconditional Active   |  YES   | *2         |

| CFU_OFF    | Call Forward Unconditional Unactive |  NO    |            |

| CFR_ON     | Call Forward On No Reply Active     |  YES   | *2#        |

| CFR_OFF    | Call Forward On No Reply Unactive   |  NO    |            |

| CFB_ON     | Call Forward On Busy Active         |  YES   | *2*        |

| CFB_OFF    | Call Forward On Busy Unactive       |  NO    |            |

| DND_ON     | Do Not Disturb Active               |  NO    |            |

| DND_OFF    | Do Not Disturb Unactive             |  NO    |            |

| VOICEMAIL  | Voice mail                          |  YES   | 0800101010 |

| PICKUPCALL | Pickup Call                         |  NO    |            |

| CR         | Call brokeRing                      |  YES   | hf 6       |

| CTB        | Call Take Back                      |  YES   | hf 6       |

| CH         | Call Hold                           |  YES   | hf 1       |

| CE         | Call Enquiry                        |  YES   | hf 1       |

| CF         | Call conFerence                     |  NO    |            |

| CWA        | Call Waiting Accept                 |  YES   | hf 6       |

| CWR        | Call Waiting Reject                 |  NO    |            |

| CWE        | Call Waiting End                    |  NO    |            |

| CFR        | Call conFerence Remove              |  NO    |            |

| CCNR       | Call Completion on No Response      |  YES   | *41        |

| CCBS       | Call Completion on Busy Subscriber  |  YES   | *42        |

| FLASH      | Flashing                            |  YES   | hf         |

+------------+-------------------------------------+--------+------------+

MXP>

These values are automatically modified if Xstate and X-features package is supported by call manager (see ref[1]). « Unknown features»  represents features which are transparently managed by the phone,  but not known basically, and which are given by the call manager through X-features.

To change any activation code, use the command "set features".

For the time being, the following services can be configured :

MXP>set features

Use one of the following commands(hookflash set parameter value is MANDATORY)

  set features <name> <hookflash[TRUE/FALSE]> [sequence]

  set features remove all

  set features remove <name>

     with <name> matching to

           1 -  CFU_ON...........Call Forward Unconditional Active

           2 -  CFU_OFF..........Call Forward Unconditional Unactive

           3 -  CFR_ON...........Call Forward On No Reply Active

           4 -  CFR_OFF..........Call Forward On No Reply Unactive

           5 -  CFB_ON...........Call Forward On Busy Active

           6 -  CFB_OFF..........Call Forward On No Reply Unactive

           7 -  DND_ON...........Do Not Disturb Active

           8 -  DND_OFF..........Do Not Disturb Unactive

           9 -  VOICEMAIL........Voice mail

           10 - PICKUPCALL.......Pickup Call

           11 - CR...............Call brokeRing

           12 - CTB..............Call Take Back

           13 - CH...............Call Hold

           14 - CE...............Call Enquiry

           15 - CF...............Call conFerence

           16 - CWA..............Call Waiting Accept

           17 - CWR..............Call Waiting Reject

           18 - CWE..............Call Waiting End

           19 - CFR..............Call conFerence Remove

           20 - CCNR.............Call Completion on No Response

           21 - CCBS.............Call Completion on Busy Subscriber

           22 - FLASH............Flashing

  set features new <id[1..20]> "name" <info[INFO/NOINFO]> <confirmation[CONF/NOCONF]> <hookflash[TRUE/FALSE]> <[sequence|NOSEQ]>

Examples :

MXP>set features CH TRUE 1
OK

MXP>

This command means that on "call hold", the phone will notify "hf,1" to the call agent.

MXP>set features CH FALSE 1
OK

MXP>

This command means that on "call hold", the phone will notify "1" to the call agent.

MXP>set features CH TRUE
OK

MXP>

This command means that on "call hold", the phone will notify "hf" to the call agent.

There is also the possibility to enter start and end sequences in the features configuration and to add a 500 ms pause. Feature sequence configuration must accept to insert information (for CF,...) in the middle of the sequence by a '$' caractère. 'P' means that a pause of 500 ms must occur.

Example: *51P$P#P#

This means that on service activation, the following sequence will be dialled : *51 then 500ms of pause, then number dialled by the user, then 500ms of pause, then #, then 500ms of pause, then #.

To save new configuration into flash, enter command « commit ».


Each configured feature can be activated into some specific state of the communication. To check the actual configuration, enter the following command :

MXP>show service_state

+--------------------+---+---+---+---+---+---+---+---+---+---+---+---+---+

|  +       STATES    | I | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 |

|      +             | D | R | D | C | A | B | H | R | C | D | C | A | B |

|          +         | L | I | I | A | C | U | O | I | W | I | A | C | U |

|              +     | E | N | A | L | T | S | L | N | T | A | L | T | S |

|  FEATURES      +   | E | G | L | L | I | Y | D | G |   | L | L | I | Y |

+--------------------+---+---+---+---+---+---+---+---+---+---+---+---+---+

| CFU_ON             | X |   |   |   |   |   |   |   |   |   |   |   |   |

| CFR_ON             | X |   |   |   |   |   |   |   |   |   |   |   |   |

| CFB_ON             | X |   |   |   |   |   |   |   |   |   |   |   |   |

| DND_ON             | X |   |   |   |   |   |   |   |   |   |   |   |   |

| VOICEMAIL          | X |   | X |   |   |   |   |   |   | X |   |   |   |

| PICKUPCALL         | X |   | X |   |   |   |   |   |   | X |   |   |   |

|                    |   |   |   |   |   |   |   |   |   |   |   |   |   |

|                    |   |   |   |   |   |   |   |   |   |   |   |   |   |

+--------------------+---+---+---+---+---+---+---+---+---+---+---+---+---+

MXP>

To associate a given feature to a given state, use command :

MXP>set service_state
Use one of the following commands:

  set service_state <state name> <service_state[KNOWN|NEW]> <feature value[1..20|..]>

     with <state name> matching to

          IDLE....................IDLE state

          ONE_RINGING_LINE........One RINGING line

          ONE_DIALING_LINE........One DIALING line

          ONE_CALLING_LINE........One CALLING line

          ONE_ACTIVE_LINE.........One ACTIVE line

          ONE_BUSY_LINE...........One BUSY line

          ONE_HOLD_LINE...........One HELD line

          TWO_LINES_RINGING.......One line in RINGING + one line RINGING

          TWO_LINES_CWT...........One line ACTIVE     + one line RINGING

          TWO_LINES_DIALING.......One line in HOLD    + one line DIALING

          TWO_LINES_CALLING.......One line in HOLD    + one CALLING line

          TWO_LINES_ACTIVE........One line in HOLD    + one line ACTIVE

          TWO_LINES_BUSY..........One line in HOLD    + one line BUSY

MXP>
The "service_state" parameter which can be "KNOW" or "NEW" is related in accordance to the features configured through "set features" command. The feature value corresponds to the number identifier of the feature.


These commands need "activate" + "commit" to be taken into account and saved into flash memory.

Setting function keys

Each key between 0 and 9 and between F1 to F4 can be used as function key. To check the current configuration of Function keys, enter the following command :

MXP>show functionkey

function key F1: Type = Function - Type = VM

function key F2: Type = Function - Type = CFR to number 150

function key F3: Type = Function - Type = FLASH

function key F4: Type = Function - Type = DND

function key  0: Type = None

function key  1: Type = Free Number - name = Mr Smith - number = 152 

function key  2: Type = None

function key  3: Type = None

function key  4: Type = None

function key  5: Type = None

function key  6: Type = None

function key  7: Type = None

function key  8: Type = None

function key  9: Type = None

MXP>

To configure function keys, use the following command :

MXP>set functionkey

To set a free number on a key

    set functionkey <ID[F1|0..9]> FREE_NUMBER "<Name>" "<Number>"

To set a function on a key

    set functionkey <ID[F1|0..9]> FUNCTION <FunctionID[DND|VM|FLASH]>

    set functionkey <ID[F1|0..9]> FUNCTION <FunctionID[CFU|CFB|CFR|PICKUP]> <Number>

To unuse a key

    set functionkey <ID[F1|0..9]> NONE

To delete a key

    set functionkey DELETE <ID[F1|0..9]>

    set functionkey DELETE ALL

 MXP>

Example :

MXP> set functionkey 1 FREE_NUMBER "Mr Smith" "152"

MXP> set functionkey F0 FUNCTION CFR 150

OK

MXP>activate

OK

MXP>commit

Check Datas integrity DATAS_HDR_NMM_DATAS...OK

OK

MXP>

RFC3149 configuration commands

To check the current RFC3149 parameters configuration, enter the command "show rfc3149":

MXP>show rfc3149

+------------+------------+

| RFC3149 FK_Value param. |

+------------+------------+

|   Key ID   | FK Number  |

+------------+------------+

|  F1        | FK1        |

|  F2        | FK2        |

|  F3        | FK3        |

|  F4        | FK4        |

|  PB        | FK92       |

|  FUNC      | inactive   |

|  DOWN      | inactive   |

|  L0        | FK99       |

|  L1        | inactive   |

|  L2        | inactive   |

|  L3        | inactive   |

|  L4        | inactive   |

|  L5        | inactive   |

|  L6        | inactive   |

|  L7        | inactive   |

|  L8        | inactive   |

|  L9        | inactive   |

+------------+------------+

+------------+------------+------------+

|     RFC3149 FKI_Value parameters     |

+------------+------------+------------+

|  FK State  |   Color    | Blink State|

+------------+------------+------------+

|    en      |   RED      | PERM LIT   |

|    db      |   NO       | UNLIT      |

|    id      |   NO       | UNLIT      |

|    dt      |   GREEN    | PERM LIT   |

|    cn      |   GREEN    | PERM LIT   |

|    dc      |   RED      | BLINK FAST |

|    rg      |   GREEN    | BLINK FAST |

|    rb      |   GREEN    | PERM LIT   |

|    ho      |   GREEN    | BLINK SLOW |

|    he      |   GREEN    | BLINK FAST |

+------------+------------+------------+

MXP>

To change any rfc3149 parameters, use the command "set rfc3149".

MXP>set rfc3149

COMMAND TO SET RFC3149 PARAMETERS:

  set rfc3149 FK_Value [F1..F4|PB|FUNC|DOWN|L0..L1] [0(no FK)|1..99(for Call Agent)|(100..199)For local Functions]

  set rfc3149 FKI_Value [string_state] [led color value] [led state value]

  set rfc3149 FKI_Value help

MXP>
The feature key value configuration

Each feature key is configurable. You can associte a key to an FK number. If you type the following command :

set rfc3149 FK_Value F1 56

in this case, each time the F1 key is pressed, the FK56 is sent to the call agent. 

To deactivate the feature type the following command:

set rfc3149 FK_Value F1 56

To use local menu with local functionality, as phone directory, use the following ID:


100 for the IP 10 configuration Menu


110 for the IP Phone Directories menu.
Example:

set rfc3149 FK_Value F1 100

The key state value configuration

Each key state appearence is configurable. You can associte a key state to a color and a blinking state. Type the following command, to see the details :

MXP>set rfc3149 FKI_Value

set rfc3149 FKI_Value [string_state] [led color value] [led state value]

string_state:

        en, db, id, dt, cn, dc, rg, rb, ho, he,

led color value:

        0 <-> COLOUR_OFF,

        1 <-> RED,

        2 <-> GREEN,

        3 <-> ORANGE

led state value:

        0 <-> UNLIT,

        1 <-> FLASH,

        2 <-> BLINK_SLOW

        3 <-> BLINK_FAST,

        4 <-> PERM_LIT

OK

MXP>
For example, if you type the following command :

set rfc3149 FKI_Value en 1 4
in this case, each time the F1 to F4 have the « en » state, they will be in permanent red color. 

Commands to set tone table elements and ringing tables

Each tone is programmable. A tone is composed of 6 notes. Each note is a combination of 4 frequencies with a configurable amplitude for each frequency. Enter the following command to set tones :

MXP>set tone
set tone nbElem ToneId <nb of elements[1 to 6]>

set tone element ToneId <elementID> <duration in ms> <F1> <A1> [<F2> <A2> <F3>

<A3> <F4> <A4>]

set tone default

show tone ToneId

With ToneId:

    -> BZ

Busy tone

    -> CG

Congestion tone

    -> DL

Dial tone

    -> BDL

Breaking dial tone

    -> DT

Disconnection tone

    -> OT

Off Hook Notice tone

    -> RT

Reorder tone

    -> RO

Ring Back tone

    -> WT1

Call waiting 1 tone

    -> WT2

Call waiting 2 tone

    -> WT3

Call waiting 3 tone

    -> WT4

Call waiting 4 tone

 Some tones are linked :

    -> Confirmation tone...............BDL (600 ms)

    -> Message Waiting Ind tone........BDL (infinite)

    -> Stutter Dial tone...............BDL (600 ms) + DL (15.4 s)

MXP>

Exemple :

MXP> set tone nbElem DL 1
MXP> set tone element DL 1 inf 350 -130 450 -130
	[image: image13.wmf]
	Frequencies range : 1 – 9999 [Hz]

Amplitude range : -800 to 30 [0,1dB], typically –60 for a 1 frequency tone, -130 for a 2 frequencies tone, -240 for a 3 frequencies tone.

Duration : inf = infinite


Exemple :

MXP>show tone DL

+------+----------+------+------+------+------+------+------+------+------+

| DL Tone configuration                                                   |

+------+----------+------+------+------+------+------+------+------+------+

| Elem | Duration |  F1  |  A1  |  F2  |  A2  |  F3  |  A3  |  F4  |  A4  |

+------+----------+------+------+------+------+------+------+------+------+

|   1  | infinite |  440 |  -60 |      |      |      |      |      |      |

+------+----------+------+------+------+------+------+------+------+------+

MXP>

The default tones of this configurable table are “UK” tones.


The phone allowes ringing melodies configuration. A melody can be associated to a "RTTTL" formatted ring. Different cadences and ring definitionn can be done, for MGCP protocol, depending on ringing signal (rg, rs, r0, r1, …, r7). To get the command syntax, enter :

MXP>set ringtones
COMMANDS TO SET RING TONES :

  set ringtones RTTTL <id[1..5]> <source[FTP|TFTP]> <filename>

  set ringtones RTTTL delete <all|[1..5]>

  set ringtones cadence <tone[R0..R7|RS|RG]> <sequence[1..4]> <ON(ms)> <OFF(ms)>

  set ringtones cadence DEFAULT <all|tone[R0..R7|RS|RG]>

  set ringtones melodymap <tone[R0..R7|RS|RG]> <DEFINT|DEFEXT|melodyId[1..15]> <adjust duration[TRUE|FALSE]

  set ringtones melodymap DEFAULT <all|tone[R0..R7|RS|RG]>

  set ringtones name <tone[R0..R7|RS|RG]> <DEFAULT|NOT_USED|name>

MXP>

ring cadence :

· set ringtones cadence <tone[R0..R7|RS|RG]> <sequence[1..4]> <ON(ms)> <OFF(ms)>

· set ringtones cadence DEFAULT <all|tone[R0..R7|RS|RG]>

Each ring tone is made of 4 ON-OFF sequences.

To show the new configuration execute :

· show ringtones cadence R2

+-----------------------+

| For Tone R2           |

+-----------------------+

| Duration :  180000 ms |

+-----------------------+

| SEQ |ON (ms) |OFF (ms)|

+-----------------------+

|  1  |     400|     200|

|  2  |     400|     200|

|  3  |     800|    4000|

|  4  |       0|       0|

+-----------------------+

Melody choice :

It’s possible to map for each ring tone a melody from the default or the RTTTL ones.

· set ringtones melodymap <tone[R0..R7|RS|RG]> <DEFINT|DEFEXT|melodyId[1..15]> <adjust duration[TRUE|FALSE]

DEFINT is the ring selected as internal ring from the IP10 Man Machine Interface.

DEFEXT is the ring selected as external ring from the IP10 Man Machine Interface.

Melody 1to 10 are the default melody,  melody 11 to 15 are the RTTTL ones.

·   set ringtones melodymap DEFAULT <all|tone[R0..R7|RS|RG]>

For each kind of MGCP ring signal (rg, r0,r1, …,r7, rs), it's possible to associate a name. Then this name will appear inside local mana machine interface and web interface, to associate a melody to a kind of ring. Per default, "rg" is "EXTERNAL" and other signals are "INTERNAL". The command syntax is :

MXP>set ringtones name <tone[R0..R7|RS|RG]> <DEFAULT|NOT_USED|name>
Some other commands

If you need to reset the phone to restore factory settings (except parameters changed through "set configure" command), enter the following command :

MXP>flash clean

All config info erased from flash

MXP>

Then the phone needs to be rebooted for the factory settings to be restored.

To reboot, you can make <CTRL-x> command, or enter the command "reboot" under prompt "->".

Commands under operating system prompt level

This command is for the ip10 configuration specialists

Under operating system prompt level ("->"), you can check software configuration.

To see the current version for application and boot, and general configuration of the phone, enter the following command :

-> tniViewBootRecord

******************* Telogy Boot Record *******************

BootLine=eim(0,0)host:ggmgcp_ipp.ftp e=10.3.16.118:ffff0000 h=10.3.10.31 g=0.0.0

.0 u=ip10_91 pw=ip10_91

RELEASE TNI BOOTROM: IP10 Boot v0.3.2

RELEASE TNI: R9.0.0 IPP (Build 5) MICRO: IP10 M v0.3.0 (Build3)

----------------------------------------------------------

MAC Address: 00:05:90:02:00:5c

Second MAC Address: ff:ff:ff:ff:ff:ff

DHCP: disabled

IP address: 10.3.16.118

Second IP address: 255.255.255.255

DNS IP address: 255.255.255.255

Second DNS IP address: 255.255.255.255

Default Gateway: 0.0.0.0

Second Gateway: 255.255.255.255

Default Netmask: 255.255.0.0

Second Default Netmask: 255.255.255.255

DSP type on board: ORION

DSP image path: flash

GG boot level: 2

VLAN ID: ffff

VLAN priority: ff

VLAN TX: disabled

prodTestParam: ff.ff.ff.ff.ff.ff

bootMsg: Booting...

fileToRestore: 0xFF

appFileName:

dspFileName:

TFTP Server: 0.0.0.0

INF File name: swupdate_ip10.inf

Application CFG File name: configip10.cfg

CFG File server type: 0

CFG File server address: 10.3.10.31

CFG File server login: ip10_91

CFG File server password: ip10_91

TFTP Server Name:

FTP Server Name:

FTP Server login:

FTP Server password:

Update Boot Version: 0x0052

Update App Version: 0x0055

Update Boot Phone: 0x01

Update App Phone: 0x01

Update Boot Protocol: 0x00

Update App Protocol: 0x01

Boot record size: 2092

**********************************************************

value = 0 = 0x0

->

Setting SIP parameters

Displays the general configuration of the SIP stack :

show sip

show sip_dbg

Command: show sip

Description: This command displays the general configuration of the SIP stack.

MXP>show sip

SIP General Configuration:

Transport Type : UDP

UDP Listen Port : 5060

TCP Listen Port : 5060

Registeration Proxy : 158.218.103.147:5060

Auto Registration : ON

Registration Timeout : 3600 secs

Refer Timeout : 0 secs

Outbound Proxy : OFF

Command: show sip_dbg

Description: This command displays the SIP modules debug level.

MXP>show sip_dbg
SIP Debug Configuration:

Log Options :

1. RVSIP_CORE : ERROR

2. RVSIP_CALL : OFF

3. RVSIP_TRANSACTION : DEBUG

4. RVSIP_MESSAGE : OFF

5. RVSIP_TRANSPORT : ERROR

6. RVSIP_PARSER : OFF

7. RVSIP_STACK : OFF

8. RVSIP_MSGBUILDER : OFF

9. RVSIP_AUTHENTICATOR : INFO

10. RVSIP_REGCLIENT : OFF

11. RVSIP_SUBSCRIPTION : DEBUG

Entering set sip produces the following command list:

MXP>set sip
set sip transport <tcp|udp>

set sip udp_listen_port <default | <port>>

set sip tcp_listen_port <default | <port>>

set sip registrar_proxy on <xxx.xxx.xxx.xxx | host_name> <port>

set sip registrar_proxy off

set sip auto_register <on|off>

set sip outbound_proxy on <xxx.xxx.xxx.xxx | host_name> <port>

set sip outbound_proxy off

set sip register_timeout <seconds>

set sip refer_timeout <seconds>

Command: set sip registrar_proxy on

Defintion: Configures the registration proxy with which the SIP endpoint registers. This command

replaces the set tcid 0 sip proxy <xxx.xxx.xxx.xxx> <port> command.

Parameter(s): <xxx.xxx.xxx.xxx | host name> <port>

Example: MXP>set sip registrar_proxy on 158.218.104.33 5060

Command: set sip registrar_proxy off

Definition: Turns off the registration proxy.

Parameter(s):

Example: MXP>set sip registrar_proxy off

Command: set sip outbound_proxy on

Definition: Configures the outbound proxy IP address and port number. If outbound proxy is on, all the signaling messages go the outbound proxy.

Parameter(s): <xxx.xxx.xxx.xxx | host name> <port>

Example: MXP>set sip outbound_proxy on 158.218.104.32 6040

Command: set sip outbound_proxy off

Definition: Turns off the outbound proxy.

Example: MXP>set sip outbound_proxy off

Command: set sip transport

Definition: Configures the transport layer for SIP.

Parameter(s): <upd | tcp>

Example: MXP>set sip transport udp

Command: set sip udp_listen_port

Definition: Configures the UDP listening port for SIP. When set to default, the port number is configured

to 5060.

Parameter(s): <default | port>

Example: MXP>set sip udp_listen_port 7050

Command: set sip tcp_listen_port

Definition: Configures the TCP listening port for SIP. When set to default, port number is configured to 5060. When the transport protocol is configured to UDP, the IP phone can still receive calls on TCP port.The converse is also true.

Parameter(s): <default | port>

Example: MXP>set sip tcp_listen_port 7050

Command: set sip auto_register

Definition: Sends REGISTER message on boot up. When configured to OFF, REGISTER message is

sent before INVITE or register_timeout expires, whichever is earlier.

Parameter(s): <on | off>

Example: MXP>set sip auto_register on

Command: set sip register_timeout

Definition: Defines the Registration expiry timer. On timeout, the IP phone resends the registration

message. IP Phone sets expiration timer to the value it sees in proxy response for the registration.

Parameter(s): <sec>

Example: MXP>set sip register_timeout 300

Command: set sip refer_timeout

Definition: Adds more flexibility to the IP Phone in handling the REFER messages. By default, the IPP toolkit uses the older version of the REFER message for the transfer feature. If no response is received for the REFER message within the specified timeout, the older version of REFER message is sent. The old and new versions of REFER message are differentiated by Refer--To header in the REFER message.

The older version of REFER uses the Refer--To header without angle brackets; for example,

Refer-To: sip:1530@157.219.123.97:5060

The new version uses angle brackets in the Refer--To header; for example,

Refer-To:<sip:1530@157.219.123.97:5060>

Parameter(s): <sec>

Example: MXP>set sip refer_timeout 5

Set SIP Debug Commands:

Command: : set sip_dbg log <all| core| call| trsn| mesg| trsp| pars| stak| msgb| auth|

rgcl| subs> <0--5(0:off,1:dbg,2:info,3:warn,4:err,5:xcpn,6:all)>

Definition: Enables debug logging of individual or all IPP toolkit & SIP stack modules with the

specified level. The level specifies the type of logging, such as general information, errors, debug

information, exceptional errors and warnings. Level set to ”off” turns off all logs. Module set to ”all”

enables logs for all modules at the specified level.

Parameter(s): <all| core| call| trsn| mesg| trsp| pars| stak| msgb| auth| rgcl| subs>

<0--5(0:off,1:dbg,2:info,3:warn,4:err,5:xcpn)>

Example: MXP>set sip_dbg log all 3

IP Phone SIP TCID Configuration Commands:

Command: set tcid 0 sip endpoint_name

Definition: This command sets the name for an endpoint used in the user field of the SIP URI.

It is mandatory that this parameter be set to receive call.

Parameter(s): <name>

Example: set tcid 0 sip endpoint_name 12345

Command: set tcid <tcid> sip auth <on|off> <user name> <password>

Definition: The IP Phone can register a unique user name and password with the proxy

server.

Example: set tcid 0 sip auth on user user123

Setting overall parameters (SIP version)

Displays the configuration of RVCM:

show rvcm

Command: show rvcm

Example: MXP>show rvcm
set rvcm dtmf_inb_pt

set rvcm nat_public_addr <xxx.xxx.xxx.xxx[:port]> 

set rvcm rtp_port <port>

set rvcm stun_addr <xxx.xxx.xxx.xxx[:port] or DNS name> 

set rvcm nat_disco <true|false>

Command: set rvcm dtmf_inb_pl <payload>

Definition: set RFC2833 inband payload.

Parameter(s): <payload>

Example: MXP>set sip rvcm_dtmf_inb_pl 101

Command: set rvcm nat_public_addr <xxx.xxx.xxx.xxx[:port]>
Definition: set gateway public address

Parameter(s): <xxx.xxx.xxx.xxx | host name :[port]>

Example: MXP>set rvcm nat_public_addr

Command: set rvcm rtp_port <port>
Definition: set port used to received RTP packets.

Parameter(s): <port>

Example: MXP>set rvcm rtp_port
Command: set rvcm stun_addr <xxx.xxx.xxx.xxx[:port]>
Definition: set STUN server address(not still available)

Parameter(s): <xxx.xxx.xxx.xxx | host name :[port]>

Example: MXP>set rvcm stun_addr

Command: set rvcm nat_disco <true/false>
Definition: enable or desable NAT discovery through STUN(not still available)

Parameter(s): <true/false>

Example: MXP>set rvcm nat_disco false
Setting H323 parameters

Gatekeeper registration mode :

To check the current registration mode, enter the following command "show rvh323" :

MXP>show rvh323
H323 General Configuration:

        Gatekeeper mode                :MANUAL

        Gatekeeper Address             :192.168.0.1:1719

        Signaling port                 :1720

        in_fast_start                  :ON

        out_fast_start                 :ON

        H.323 Aliases:

                (E164) 220

                (H323) ip10-220

Gatekeeper registration mode can be changed with the command :

"set rvh323 gk_mode <off | manual | auto>":

MXP>set rvh323 gatekeeper manual

Default configuration is manual, that allow you to configure the IP address of the gatekeeper. Automatic mode launches an automatic discovery of the gatekeeper using multicast address 224.0.1.41. Discovery mode off means that the phone won’t register to a gatekeeper. 

Gatekeeper localization :

To check the current gatekeeper localization, enter the following command "show rvh323" :

MXP>show rvh323
H323 General Configuration:

        Gatekeeper mode                :MANUAL

        Gatekeeper Address             :192.168.0.1:1719

        Signaling port                 :1720

        in_fast_start                  :ON

        out_fast_start                 :ON

        H.323 Aliases:

                (E164) 220

                (H323) ip10-220

Gatekeeper IP address can be changed with the command :

"set rvh323 gatekeeper <Ip Address:port>":

MXP>set rvh323 gatekeeper 192.168.0.1:1719
The port number is optional, default port number is 1719. This port is only used for registration to the gatekeeper.

Gatekeeper address isn’t applicable when discovery mode is automatic. 

When discovery mode is off, the gatekeeper address is the address of the signaling server.

Fast start :

To check the fast start activation, enter the following command "show rvh323" :

MXP>show rvh323
H323 General Configuration:

        Gatekeeper mode                :MANUAL

        Gatekeeper Address             :192.168.0.1:1719

        Signaling port                 :1720

        in_fast_start                  :ON

        out_fast_start                 :ON

        H.323 Aliases:

                (E164) 220

                (H323) ip10-220

Using fast start procedure for outgoing calls can be configured with the command :

"set rvh323 out_fast_start <0|1|on|off>":

MXP>set rvh323 out_fast_start on
The in_fast_start value isn’t applicable. In any case, if an incomming call uses the fast start procedure, the IP10S won’t refuse it (when it’s possible). 

Phone aliases

The H323 protocol defines 2 types of aliases :

-E164 alias: This is a the phone number of the phone according to the international dialing plan

-H323 Id:
This is a string that describes the phone

To check the phone aliases, enter the following command "show rvh323" :

MXP>show rvh323
H323 General Configuration:

        Gatekeeper mode                :MANUAL

        Gatekeeper Address             :192.168.0.1:1719

        Signaling port                 :1720

        in_fast_start                  :ON

        out_fast_start                 :ON

        H.323 Aliases:

                (E164) 220

                (H323) ip10-220

H323 id can be managed with the commands :

"set rvh323  alias add <alias>"

"set rvh323  alias delete <alias>"

"set rvh323  alias delete all"

MXP>set rvh323 alias delete all
MXP>set rvh323 alias add 220
In order to keep only one E164 alias in the list, we advise you to type command “delete all” before adding a new alias.

E164 aliases can be managed with the commands :

"set rvh323 id add "h323 Id" "

"set rvh323 id delete "h323 Id" "

"set rvh323 id delete all"

MXP>set rvh323 id delete all
MXP>set rvh323 id add “IP10 220”
In order to keep only one H323 id in the list, we advise you to type command “delete all” before adding a new alias.

Tunneling :

Tunneling parameter can be changed with the command :

"set rvh323 tunneling <0|1|on|off>":

MXP>set rvh323 tunneling on
Setting overall parameters (H323 version)

DTMF relay :

Ip10S can send DTMF tones over the network using 3 different modes:

DTMF relay off: DTMF tones are sent in the voice stream.

DTMF relay in band: DTMF tones are sent in the RTP stream using RFC 2833

DTMF relay out of band: DTMF tones are sent as H245 User Input Indication

To check the DTMF relay mode, enter the following command "show rvcm" :

MXP>show rvcm
DTMF Relay            :OOB

        UI Term Name          :ip10-220

        UI Term Phone Number  :220

        Numbering infos:

        ================

        cdp_number=

        location_code=

        area_code=

        country_code=

        did=0

DTMF relay mode can be configured with the command :

"set rvcm dtmf_relay <off|inb|oob>":

MXP>set rvh323 dtmf_relay oob
Inb means In Band DTMF relay (RFC 2833).

Oob means Out Of Band DTMF relay (H245 signaling). 

H323 debug traces :

You can obtain H323 debug traces using the following commands

"set rvcm log_level <0|1|2|3>" 

"set rvcm log_source <log_source> <on|off>" 

Log level 0 disable all traces.

Log level 1 is a non-verbose debug level for selected sources 

Log level 2 is a verbose debug level for selected sources

Log level 3 is a verbose debug level for all sources

To obtain full list of log sources, type command :”show rvcm log_source all”

To obtain the list of selected sources, type command : “show rvcm log_source”

Main log  sources are:

UDPCHAN to see UDP messages

TPKTCHAN to see TCP messages

CMAPICB and CMAPI to get the stack of called functions

Setting codec parameters (H323 and SIP versions)

Codec priority :

To check the current codec priorities, enter the following command "show rtp_prof" :

MXP>show rtp_prof
Configuration for coding profile id 0:

        codec type  = G.711 MU

        packet time = 30 msec

        priority    = 254

        max Jitter  = 120 msec

        vad         = 1

Configuration for coding profile id 1:

        codec type  = G723 6.3 kbps

        packet time = 60 msec

        priority    = 255

        max Jitter  = 240 msec

        vad         = 1

Configuration for coding profile id 2:

        codec type  = G.711 A

        packet time = 30 msec

        priority    = 253

        max Jitter  = 120 msec

        vad         = 1

Configuration for coding profile id 3:

        codec type  = PACKET SIGNALING

        packet time = 0 msec

        priority    = 0

        max Jitter  = 0 msec

        vad         = 0

Highest priority value is 255, lowest priority value is 1. 0 is an invalid priority

You can set priorities by typing command :

“set rtp_prof  <prof_id>  priority <1.....255(0:invalid, 1:lowest, 255:highest)>”

Don’t leave 2 profiles with the same priorities.

Maximum jitter :

To check the current maximum jitter, enter the following command "show rtp_prof" :

MXP>show rtp_prof
Configuration for coding profile id 0:

        codec type  = G.711 MU

        packet time = 30 msec

        priority    = 254

        max Jitter  = 120 msec

        vad         = 1

Configuration for coding profile id 1:

        codec type  = G723 6.3 kbps

        packet time = 60 msec

        priority    = 255

        max Jitter  = 240 msec

        vad         = 1

Configuration for coding profile id 2:

        codec type  = G.711 A

        packet time = 30 msec

        priority    = 253

        max Jitter  = 120 msec

        vad         = 1

Configuration for coding profile id 3:

        codec type  = PACKET SIGNALING

        packet time = 0 msec

        priority    = 0

        max Jitter  = 0 msec

        vad         = 0

You can set max jitter for each codec by typing command :

“set rtp_prof  <prof_id> max_jitter <size(msec)>”

The maximum values are 390msec for g729 and g723 and 120msec for g711.

Voice activity detection :

To check the voice activity detection, enter the following command "show rtp_prof" :

MXP>show rtp_prof
Configuration for coding profile id 0:

        codec type  = G.711 MU

        packet time = 30 msec

        priority    = 254

        max Jitter  = 120 msec

        vad         = 1

Configuration for coding profile id 1:

        codec type  = G723 6.3 kbps

        packet time = 60 msec

        priority    = 255

        max Jitter  = 240 msec

        vad         = 1

Configuration for coding profile id 2:

        codec type  = G.711 A

        packet time = 30 msec

        priority    = 253

        max Jitter  = 120 msec

        vad         = 1

Configuration for coding profile id 3:

        codec type  = PACKET SIGNALING

        packet time = 0 msec

        priority    = 0

        max Jitter  = 0 msec

        vad         = 0

Voice activity detection allow to reduce bandwidth use by using silence compression algorithms.

You can set voice activity detection for each codec by typing command :

“set rtp_prof  <prof_id> vad <on|off>”

DTMF relay config(only with SIP):

To check the dtmf configuration in SIP,enter the following command "show rtp_prof":

MXP>show rtp_prof
Configuration for coding profile id 0:

        codec type  = G.711 MU

        packet time = 30 msec

        priority    = 254

        max Jitter  = 120 msec

        vad         = 1

        DTMF Relay  = INBAND

        DTMF Digit Payload = 101

Configuration for coding profile id 1:

        codec type  = G723 6.3 kbps

        packet time = 60 msec

        priority    = 255

        max Jitter  = 240 msec

        vad         = 1

        DTMF Relay  = INBAND

        DTMF Digit Payload = 101

Configuration for coding profile id 2:

        codec type  = G.711 A

        packet time = 30 msec

        priority    = 253

        max Jitter  = 120 msec

        vad         = 1

        DTMF Relay  = INBAND

        DTMF Digit Payload = 101

Configuration for coding profile id 3:

        codec type  = PACKET SIGNALING

        packet time = 0 msec

        priority    = 0

        max Jitter  = 0 msec

        vad         = 0

        DTMF Relay  = INBAND

        DTMF Digit Payload = 101

Set rtp_prof DTMF commands:

Command: set rtp_prof dtmf_relay <oob|inb|off> [payload]

Defintion: This command selects the DTMF Relay mode, in which DTMF tones are detected during

voice processing and packetized separately for transmission. The DTMF relay setting is applicable to all coding profiles. The optional dynamic payload type will be used for in band DTMF relay mode. If

dynamic payload is not configured for in band, the default payload type 97 will be used.

Parameter(s): <oob|inb|off> [payload]

Example: set rtp_prof dtmf_relay inb 101

DTMF inband payload configured in rtp_prof must also be reported in RVCM configuration through this command:

Command: set rvcm dtmf_inb_pt <payload>

Setting numbering table (H323 and SIP versions)

Data access :

In order to modify data of the numbering table, you have to access the control mode. To enter the control mode, enter the following command "atpm req" :

MXP>atpm req

OK

To leave the control mode, enter the following command "atpm done" :

MXP>atpm done

OK

To save data into flash memory, enter the following command "atpm store" :

MXP>atpm store

OK

Address list :

To see current address list, enter the following command "atpm alist" :

MXP>atpm alist

                       Address  Hunt   Min    Max    Prefx Prefix

                       Entry    Grp_Id Digits Digits strip Address

                             0     10    1     26      0   None

                             1     10    1     26      0   None

                             2     10    1     26      0   None

                             3     10    1     26      0   None

                             4     10    1     26      0   None

                             5     10    1     26      0   None

                             6     10    1     26      0   None

                             7     10    1     26      0   None

                             8     10    1     26      0   None

                             9     10    1     26      0   None

                             *     10    1     26      0   None

                             #     10    1     26      0   None

OK

To add a new address in the list, enter the following command:

"atpm aadd <number> <min_digits> <max_digits> <hunt_grp_id> <prefix_strip_len> [prefix_number]" :

MXP>atpm aadd 0 1 26 10 0

OK

To delete an entry in the list, enter the following command:

"atpm adel <number>" :

MXP>atpm adel 0

OK


To add a destination address associated with user id(alias) :

“atpm dadd <dest_id> rvcm <addr>”:

H323:

        addr -> <TA:<ip_addr_dest <NAME:<endpoint_name>|TEL:<endpoint_number>>

SIP:

        addr -> <[user@]ip_addr_dest|ip_addr_proxy[:port]>

    MXP>atpm dadd 83 rvcm 22@10.5.10.2

    OK

ANNEXE A:configip10.cfg sample for SIP

set tcid 0 sip endpoint_name 21

set sip registrar_proxy on 10.5.17.10 5060

set tcid 0 sip auth on 21 swissvoice

set sip transport udp

set rtp_prof 0 vad off

set rtp_prof 1 vad off

set rtp_prof 2 vad off

set features VOICEMAIL 15

set functionkey DELETE ALL

set functionkey F0 FUNCTION VM

set functionkey F1 FREE_NUMBER "Pickup" "*8"

set utilities sntp_mode request

set utilities sntp_srv_ipaddr 172.17.100.111

set utilities sntp_time_zone 12

set utilities sntp_rqnt_freq 1 hh 0 mm

set rvcm dtmf_relay inb 101

set telbook del internal

set telbook add internal "Damien Sandras" "6001"

set telbook add internal "Damien GnomeMeeting" "6005"

set telbook add internal "6008" "6008"

set telbook add internal "Damien Grandstream" "6002"

set telbook add internal "Xten" "6012"

set telbook add internal "pstn phone" "6009"

set telbook add internal "damien kphone" "6007"

set telbook add internal "damien kphone" "6011"

set telbook add internal "6006" "6006"

set telbook add internal "My CellPhone 2" "6008"

set telbook add internal "Groupe Technique" "71"

set telbook add internal "Groupe cosmique" "72"

activate

commit

www.swissvoice.net
Swissvoice S.A. 2003

Subject to modifications















Telnet Commands
1
AG0803Aen

